# Exercise 01:

To which class do the following addresses belong?

- 142.26.67.89 B Class

- 171.12.15.78 B Class

- 12.15.4.55	A Class

- 192.24.76.123	C Class

- 221.55.67.123	C Class

- 123.86.78.23	A Class

- 126.19.76.23	A Class

# Exercise 02:

In this exercise, you need to determine how many bits are required to create the requested number of subnets.

- 84 subnets	7 bits (27 = 128)

- 145 subnets 8 bits (28 = 256)

- 7 subnets	3 bits (23 = 8)

- 1 subnet	1 bits (21 = 2)

- 15 subnets	4 bits (24 = 16)

# Exercise 03:

From a network ID and a desired number of subnets, calculate the subnet mask and the number of hosts per subnet.

- Network ID: 148.25.0.0 and 37 subnets

The subnet ID belongs to Class B. For 37 subnets, we need to allocate 6 bits (26 = 64). The subnet mask is therefore: 255.255.11111100.0, or 255.255.252.0. This leaves 10 bits (2 + 8) for the host

identifier. Thus, there are 1022 machines per subnet (210 - 2 = 1022).

- Network ID: 198.63.24.0 and 4 subnets

The subnet ID belongs to Class C. For 4 subnets, we need to allocate 2 bits (22 = 4). The subnet mask is therefore: 255.255.255.11000000, or 255.255.255.192. This leaves 6 bits for the host identifier. Thus, there are 62 machines per subnet (26 - 2 = 62).

- Network ID: 110.0.0.0 and 1000 subnets

The subnet ID belongs to Class A. For 1000 subnets, we need to allocate 10 bits (210 = 1024). The subnet mask is therefore: 255.11111111.11000000.0, or 255.255.192.0. This leaves 14 bits (6 + 8) for the host identifier. Thus, there are 16,382 machines per subnet (214 - 2 = 16,382).

- Network ID: 175.23.0.0 and 550 subnets

The subnet ID belongs to Class B. For 550 subnets, we need to allocate 10 bits (210 = 1024). The subnet mask is therefore: 255.255.11111111.11000000, or 255.255.255.192. This leaves 6 bits for the host identifier. Thus, there are 62 machines per subnet (26 - 2 = 62).

- Network ID: 209.206.202.0 and 60 subnets

The subnet ID belongs to Class C. For 60 subnets, we need to allocate 6 bits (26 = 64). The subnet mask is therefore: 255.255.255.11111100, or 255.255.255.252. This leaves 2 bits for the host identifier. Thus, there are 2 machines per subnet (22 - 2 = 2)

# Exercise 04:

Suppose there is a network with the IP address 195.180.125.0. You need at least 22 subnets.

- How many bits do you need to borrow? We need to borrow: 5 bits (25 = 32).

- What is the total number of subnets available? The total number of subnets is: 25 = 32.

- How many hosts per subnet do you have?

. The number of hosts per subnet is: 23 - 2 = 6 hosts.

- What is the subnet mask?

The subnet mask is: 255.255.255.248.

- Provide the range of available host addresses on the first usable subnet. The range of available host addresses in the first usable subnet is:

Subnet 1: 194.170.25.1 — 194.170.25.6 (/29)

# Exercise 05:

A computer P1 with the following network parameters:

IP Address: 200.100.10.60

Subnet Mask: 255.255.255.224

- Encode the IP address in binary.

The binary equivalent of the IP address is: 11001000.01100100.00001010.00111100.

- What is the class of the network?

This IP address is a Class C address. (Class C address range: 192.0.0.0 --- 223.255.255.255)

- What is the subnet address? The subnet address is:

11001000.01100100.00001010.00111100 (200.100.10.60)

AND 11111111.11111111.11111111.11100000 (255.255.255.224)

11001000.01100100.00001010.00100000 (200.100.10.32)

Therefore, the subnet address is: 200.100.10.32.

- What is the broadcast address for this subnet?

The broadcast address of this subnet is: 11001000.01100100.00001010.00111111 (200.100.10.63).

- Does the IP address 200.100.10.35 belong to this subnet?

11001000.01100100.00001010.00100011 (200.100.10.35)

AND 11111111.11111111.11111111.11100000 (255.255.255.224)

11001000.01100100.00001010.00100000 (200.100.10.32)

Therefore, the IP address 200.100.10.35 belongs to the subnet 200.100.10.32 /27.

# Exercise 06:

Consider the following IP address:

172.16.5.32 /24

- Provide the subnet mask. The subnet mask is: 255.255.255.0

- Write this mask in binary.

The mask in binary is: 11111111.11111111.11111111.00000000.

- What is the class of the network?

This IP address is a Class B address. (Class B address range: 128.0.0.0 --- 191.255.255.255).

- What is the subnet address? The subnet address is:

10101100.00010000.00000101.00100000

AND 11111111.11111111.11111111.00000000

10101100.00010000.00000101.00000000

Therefore, the subnet address is: 172.16.5.0 /24.

- What is the broadcast address for this subnet?

The broadcast address of this subnet is: 10101100.00010000.00000101.11111111 (172.16.5.255).

# Exercise 07:

You work in a medical office. All the computers in the office are networked as shown in the illustration in Appendix A. We are dealing with a network using the TCP/IP protocol. The IP addresses of each node in the network are listed in Appendix B. For all of them, the default mask is 255.255.255.0.

| Computer | IP address |
| --- | --- |
| PC1 | 192.168.10.6 |
| PC2 | 192.168.10.7 |
| PC3 | 192.168.10.8 |
| PC4 | 192.168.10.9 |
| Server 1 | 192.168.10.100 |
| Server 2 | 192.168.10.200 |

# Appendix A

- What is the architecture of this network?

The architecture of this network is: star network.

- Indicate what the IP address of the network is. The IP address of the network is: 192.168.10.0 /24.

# Appendix B

- Determine the number of machines that can be connected to this network. This network can contain 28 - 2 = 254 hosts.

- What is the broadcast address of this network?

The broadcast address of this network is: 192.168.10.255.

- What are the different subnets obtained if the 4th octet of the new mask is: 11000000? The subnets are as follows:

  - 192.168.10.0 /26

  - 192.168.10.64 /26

  - 192.168.10.128 /26

  - 192.168.10.192 /26.

# Exercise 08:

Suppose there is an IP network with the address 194.170.25.0. You need at least 20 subnets with at least 5 (usable) hosts per subnet.

- How many bits do you need to borrow? We need to borrow: 5 bits (25 = 32).

- What is the total number of subnets you will have? The total number of subnets is: 25  = 32.

- How many hosts per subnet do you have?

The number of hosts per subnet is: 23 - 2 = 6 hosts.

- What is the subnet mask?

The subnet mask is: 255.255.255.248.

- Provide the range of available host addresses on the first usable subnet.

The range of available host addresses on the first usable subnet: Subnet 1: 194.170.25.1 --- 194.170.25.6 (/29).

# Exercise 09:

Suppose there is an IP network with the address 192.52.0.0. You borrow 4 bits to create subnets.

- What is the class of this IP address? This address is a Class C address.

- What is the total number of subnets available?

The total number of subnets is: 24  =  16 subnets.

- How many hosts per subnet do you have?

The number of hosts per subnet is: 24 - 2 = 16 - 2 = 14 hosts.

- What is the subnet mask?

The subnet mask is: 255.255.255.240.

- Provide the range of available host addresses on the second usable subnet.

The range of available host addresses on the second usable subnet is: 192.52.0.17 --- 192.52.0.30 (/28).

# Exercise 10:

We assign the network 200.100.1.0 /24. We need to place 20 hosts in each subnet.

- How many bits are needed in the host portion of the assigned address to accommodate at least 20 hosts?

The number of bits needed to accommodate at least 20 hosts is: 5 (25 = 32).

- What is the maximum number of usable host addresses in each subnet? The maximum number of usable host addresses in each subnet is: 25 - 2 = 30.

- What is the maximum number of defined subnets? The maximum number of defined subnets is: 23 = 8.

- What are the addresses of all the defined subnets? The addresses of all defined subnets are:

  - Subnet 1: 200.100.1.0 (11001000.01100100.00000001.00000000)

  - Subnet 2: 200.100.1.32 (11001000.01100100.00000001.00100000)

  - Subnet 3: 200.100.1.64 (11001000.01100100.00000001.01000000)

  - Subnet 4: 200.100.1.96 (11001000.01100100.00000001.01100000)

  - Subnet 5: 200.100.1.128 (11001000.01100100.00000001.10000000)

  - Subnet 6: 200.100.1.160 (11001000.01100100.00000001.10100000)

  - Subnet 7: 200.100.1.192 (11001000.01100100.00000001.11000000)

  - Subnet 8: 200.100.1.224 (11001000.01100100.00000001.11100000)

- What is the broadcast address of subnet number 3?

The broadcast address of subnet number 3 is: 200.100.1.95.